# Annexure A — Comparative Analysis of Network Topologies

This submission compares the four fundamental network topologies — Star, Bus, Mesh, and Hybrid — across the key dimensions specified in the assessment: cost, reliability, performance, fault tolerance, installation complexity, scalability, and maintenance. Each question below states the comparison, presents a structured side-by-side analysis, and closes with a justified conclusion.

# Question 1 — Star vs. Bus Topology: Cost, Reliability, and Performance (5 Marks)

Compare star and bus topology in terms of cost, reliability, and performance.

## 1.1 Objective

To evaluate star and bus topologies against three practical deployment criteria — cost of implementation, reliability under failure, and performance under load — and to justify which topology is preferable for a given criterion.

## 1.2 Comparison Table

| Criterion | Star Topology | Bus Topology |
| --- | --- | --- |
| Cabling / Hardware | Requires a central device (switch/hub) plus a dedicated cable run to every node — higher cable and hardware cost. | Uses a single shared backbone cable with drop cables — minimal hardware, lowest material cost. |
| Installation Cost | Generally more expensive due to central device and per-node wiring. | Low-cost and quick to install for small networks. |
| Reliability | High — failure of one node or its cable does not affect any other node. | Low — a break or fault anywhere on the backbone brings down the entire segment. |
| Performance | Better — the central device manages transmission and reduces collisions. | Degrades as more devices are added, since all nodes share one transmission medium and contend for it. |
| Best Suited For | Networks needing high uptime and easy fault isolation (offices, labs). | Very small, low-budget, temporary networks. |

## 1.3 Analysis / Discussion

The cost difference is structural: star topology's dependence on a central concentrator and individual home-runs to every device multiplies cabling length and requires an active networking device, whereas bus topology's single shared trunk avoids both. This cost advantage of bus topology, however, is exactly what makes it fragile — because all devices tap into one physical medium, a single cable break, a bad terminator, or a faulty tap is a single point of failure for the whole network. Star topology inverts this trade-off: the extra cost buys fault isolation, since each node's cable and connection are independent of every other node's. Performance follows a similar pattern — bus topology's shared medium means every additional device increases the chance of collisions and reduces effective throughput, while a switch-based star topology can forward frames only to the intended port, so aggregate performance scales far better as nodes are added.

## 1.4 Conclusion

Star topology costs more to install but is markedly superior in reliability and performance because it isolates faults and manages traffic centrally. Bus topology is cheaper and simpler but sacrifices both reliability and scalable performance, making it suitable only for small, non-critical, low-cost deployments.

# Question 2 — Mesh vs. Star Topology: Fault Tolerance (5 Marks)

Compare mesh and star topology in terms of fault tolerance.

## 2.1 Objective

To assess how mesh and star topologies respond to link and device failures, and to determine which topology offers superior fault tolerance for mission-critical environments.

## 2.2 Comparison Table

| Criterion | Mesh Topology | Star Topology |
| --- | --- | --- |
| Connection Pattern | Every node connects to multiple (often all) other nodes, providing several independent paths. | Every node connects only to a single central hub/switch. |
| Effect of a Single Link Failure | Traffic is automatically re-routed through an alternate path; communication continues uninterrupted. | Only the one node on that link loses connectivity; the rest of the network is unaffected. |
| Effect of Central Device Failure | Not applicable — there is no single central device the whole network depends on. | Catastrophic — failure of the hub/switch disconnects every node simultaneously. |
| Fault Tolerance Level | Very high — redundant paths make the design robust against multiple simultaneous faults. | Moderate — resilient to node/cable faults but has one systemic single point of failure. |
| Typical Use Case | Critical backbones (ISP cores, military/defence networks, data-centre interconnects). | General-purpose LANs where moderate resilience is acceptable. |

## 2.3 Analysis / Discussion

The key distinguishing factor is redundancy of paths versus redundancy of nodes. Mesh topology's defining property — multiple physical paths between any pair of nodes — means the network has no true single point of failure at the topology level; when a link fails, routing/switching logic simply uses another available path, and communication is preserved. Star topology, in contrast, achieves resilience only at the node level: if one workstation's cable fails, the rest of the network is unaffected, which already makes it more fault-tolerant than bus topology. However, every node's only path to the network passes through the same central device, so the hub or switch is an unavoidable single point of failure — if it goes down, the entire star network stops functioning at once, regardless of how many nodes are otherwise healthy. This is why mesh topology, despite its far higher wiring and hardware cost, is preferred in environments where continuous availability is non-negotiable.

## 2.4 Conclusion

Mesh topology provides substantially higher fault tolerance than star topology because its redundant paths eliminate any single point of failure, while star topology remains vulnerable to complete network failure if its central device fails. Mesh is therefore the stronger choice for critical, high-availability environments.

# Question 3 — Bus vs. Mesh Topology: Installation Complexity (5 Marks)

Compare bus and mesh topology in terms of ease/complexity of installation.

## 3.1 Objective

To compare the wiring effort, connection count, and practical installation difficulty of bus and mesh topologies as the number of network devices increases.

## 3.2 Comparison Table

| Criterion | Bus Topology | Mesh Topology |
| --- | --- | --- |
| Cabling Required | One shared backbone cable with short drop cables to each node. | A dedicated cable between every pair of nodes. |
| Number of Connections for n Nodes | Approximately n connections to the single backbone (linear growth). | n(n − 1) / 2 connections for full mesh (quadratic growth). |
| Ports Required per Node | One tap/connector per node. | n − 1 ports per node in a full mesh — significant NIC/port demand. |
| Installation Effort | Quick and simple; minimal planning needed. | Labour-intensive; requires careful planning of cable runs and port allocation. |
| Scal­ability of Installation Effort | Adding a node is a minor, low-cost extension of the backbone. | Adding a node requires new cables to every existing node — effort grows sharply with network size. |

## 3.3 Analysis / Discussion

Bus topology's installation simplicity comes from its linear structure: adding a node means tapping into the existing backbone once, so total wiring grows proportionally to the number of nodes. Mesh topology's fault tolerance (Question 2) is bought at a steep installation cost — a full mesh of n nodes needs n(n − 1)/2 physical links, so wiring effort grows quadratically rather than linearly. For example, a 5-node bus needs about 5 connections, while a full 5-node mesh needs 10; at 10 nodes this becomes roughly 10 versus 45. Beyond the sheer cable count, mesh installations also demand more networking ports per device and far more careful physical planning (cable routing, labelling, and testing every individual link), making mesh topology considerably more time-consuming and costly to deploy, especially as the network grows.

## 3.4 Conclusion

Bus topology is simple and inexpensive to install because of its single shared backbone, while mesh topology's requirement for a direct link between every pair of nodes makes installation complex and its effort grow quadratically with network size. Bus topology suits small, simple networks; mesh topology's installation overhead is justified only where its fault tolerance is essential.

# Question 4 — Hybrid vs. Star Topology: Scalability (5 Marks)

Compare hybrid and star topology in terms of scalability.

## 4.1 Objective

To examine how hybrid and star topologies accommodate network growth, and to identify the limiting factors on scalability in each case.

## 4.2 Comparison Table

| Criterion | Hybrid Topology | Star Topology |
| --- | --- | --- |
| Structure | Combines two or more topologies (e.g. star-of-stars, star + mesh) into one design. | All nodes connect to a single central device. |
| Expansion Method | New segments (additional star/bus/mesh clusters) can be added and interconnected without redesigning the existing network. | New nodes are added directly to the existing central device. |
| Limiting Factor | Limited mainly by the capacity of the interconnecting backbone/core devices, which can themselves be upgraded or expanded. | Limited directly by the port count of the central hub/switch; once full, expansion requires additional hardware. |
| Scalability Rating | High — flexible, near-organic growth across departments or sites. | Moderate — scalable up to central-device capacity, then requires structural change. |
| Typical Use Case | Large enterprises, campus networks, multi-department organisations. | Single office, lab, or floor with a bounded number of devices. |

## 4.3 Analysis / Discussion

Star topology's scalability is bounded by a single physical constraint: the number of available ports on its central switch or hub. Growth is easy up to that limit, but once the device is full, expansion requires purchasing and integrating new hardware, and possibly restructuring cabling. Hybrid topology avoids this ceiling by design — because it is built from multiple interconnected topological segments (for instance, several star clusters joined through a core mesh or backbone link), new segments can be introduced as independent units and connected at the core, without disturbing the internal structure of existing segments. This makes hybrid topology naturally suited to organisations that grow in units — new departments, floors, or branch sites — since each new unit is simply added as another segment rather than requiring the entire network to be re-planned. The trade-off is that hybrid topology is more complex to design and requires competent network architecture at the core, but this complexity is precisely what buys its superior long-term scalability.

## 4.4 Conclusion

Hybrid topology offers greater scalability than star topology because it grows by adding interconnected segments rather than depending on a single device's port capacity. Star topology remains scalable only within the limits of its central hardware, after which further growth requires hardware upgrades or a topology change.

# Question 5 — Maintenance Comparison Across Star, Mesh, Bus, and Hybrid Topologies (5 Marks)

Compare star, mesh, bus, and hybrid topology in terms of ease of maintenance.

## 5.1 Objective

To rank all four topologies by how easily faults can be identified, isolated, and resolved, and to explain the structural reasons behind each topology's maintenance profile.

## 5.2 Comparison Table

| Topology | Maintenance Difficulty | Reason |
| --- | --- | --- |
| Star | Easy | Each node has an independent cable back to the central device, so a fault can be quickly isolated to a specific node or link without affecting diagnosis of the rest of the network. |
| Bus | Moderately Hard | All nodes share one backbone cable, so a fault anywhere on it disrupts the entire network, and locating the exact break point along a long shared cable is time-consuming. |
| Mesh | Hard | The very large number of point-to-point connections that gives mesh its fault tolerance also makes troubleshooting complex, since many redundant paths must be traced and tested individually. |
| Hybrid | Moderate | Combines multiple topologies, so flexibility is gained, but administrators must monitor and coordinate different segment types simultaneously, requiring broader skill and careful documentation. |

## 5.3 Analysis / Discussion

Maintenance difficulty tracks the same structural property that governs fault tolerance and installation complexity, but from the opposite direction: the design elements that make a topology reliable or resilient are often the elements that make it hard to diagnose. Star topology remains the easiest to maintain because its point-to-point cabling means a technician can isolate a fault to one link or node almost immediately, without disturbing any other connection. Bus topology, despite being simple to install, is comparatively harder to maintain because its single shared backbone gives no way to isolate a section — the whole segment must be tested to find one break. Mesh topology, while the most fault-tolerant of the four, is the hardest to maintain precisely because it has the most connections: verifying which of many redundant paths has failed, and confirming that failover behaved correctly, demands significant time and expertise. Hybrid topology sits in between — its flexibility does not come with a single dominant weak point, but its mixed structure means the administrator must understand and monitor each constituent topology's characteristics simultaneously, which requires broader skill even if no single fault is especially hard to trace.

## 5.4 Conclusion

Ranked from easiest to hardest to maintain: Star < Hybrid < Bus < Mesh. Star topology's isolated point-to-point links make faults simplest to find and fix; hybrid topology adds moderate coordination overhead; bus topology's shared backbone makes fault-location harder; and mesh topology's dense interconnection, though excellent for reliability, makes it the most demanding to maintain.

# Overall Summary

| Criterion | Star | Bus | Mesh | Hybrid |
| --- | --- | --- | --- | --- |
| Cost | High | Low | Very High | High |
| Reliability | High | Low | Very High | High |
| Performance | Good (managed centrally) | Degrades with load | Excellent (multiple paths) | Good, depends on design |
| Fault Tolerance | Moderate (hub is SPOF) | Low (backbone is SPOF) | Very High (no SPOF) | High |
| Installation Complexity | Moderate | Low | Very High | High |
| Scalability | Moderate (port-limited) | Low | Low (cost-limited) | Very High |
| Maintenance | Easy | Moderately Hard | Hard | Moderate |

SPOF = Single Point of Failure