CSA07 – CO5 Assessment Tool 2

Pseudocode Writing Task – Professional Answer Document

Computer Networks (CSA07) • CO5: Measure network performance using key metrics with simulation tools

The algorithms below use clear inputs, validation, control structures and outputs, matching the assessment requirement for complete and logically sound pseudocode.

Question 1: Network Throughput and Packet Loss

Given 8000 bits received in 4 seconds, 200 packets sent and 180 packets received, calculate throughput and packet loss. Validate time and packets sent, classify the network, and display a complete summary.

Pseudocode

BEGIN dataReceived ← 8000 time ← 4 packetsSent ← 200 packetsReceived ← 180 IF time <= 0 OR packetsSent <= 0 THEN DISPLAY "Error: time and packets sent must be greater than zero." STOP END IF IF packetsReceived < 0 OR packetsReceived > packetsSent THEN DISPLAY "Error: invalid received-packet value." STOP END IF throughput ← dataReceived / time lostPackets ← packetsSent - packetsReceived packetLoss ← (lostPackets / packetsSent) × 100 DISPLAY "----- NETWORK PERFORMANCE -----" DISPLAY "Throughput = ", throughput, " bps" DISPLAY "Packets Sent = ", packetsSent DISPLAY "Packets Received = ", packetsReceived DISPLAY "Packet Loss = ", packetLoss, "%" IF throughput > 1500 AND packetLoss < 10 THEN status ← "Efficient" ELSE status ← "Inefficient" END IF DISPLAY "Performance Classification = ", status DISPLAY "Summary: The network delivered ", throughput, " bps with ", packetLoss, "% packet loss." END

Explanation: Throughput = 8000/4 = 2000 bps. Packet loss = (200−180)/200 × 100 = 10%. Although the throughput is above 1500 bps, the loss is not below 10%, so the network is classified as Inefficient. The added validation prevents division-by-zero and invalid packet counts.

Question 2: Bandwidth Utilization Monitoring

Periodically collect bandwidth utilization for every network link, store the values, detect utilization above 80%, and recommend or perform traffic switching to a backup link.

Pseudocode

BEGIN links ← GetAllNetworkLinks() monitoringInterval ← 60 seconds WHILE TRUE DO FOR each link IN links DO usage ← MeasureBandwidthUtilization(link) Store usage in usageList DISPLAY link, " utilization = ", usage, "%" IF usage > 80 THEN DISPLAY "Warning: ", link, " is overloaded." IF BackupLinkAvailable(link) THEN DISPLAY "Recommend switching traffic to backup link." SwitchTrafficToBackup(link) ELSE DISPLAY "No backup link available. Alert administrator." ALERT administrator END IF END IF END FOR WAIT monitoringInterval END WHILE END

Explanation: The loop provides continuous monitoring, while the 80% threshold gives an early warning before the link becomes severely congested. Storing measurements also supports trend analysis and capacity planning. Switching traffic to a backup path reduces congestion and helps maintain service availability.

Question 3: Best-Path Selection Using Link Quality

Continuously collect bandwidth, delay and utilization for multiple paths. Calculate a quality score, choose the highest-scoring healthy path, detect failures/overload, select an alternative and alert the administrator.

Pseudocode

BEGIN paths ← GetAllPossiblePaths() monitoringInterval ← 60 seconds WHILE TRUE DO FOR each path IN paths DO bandwidth ← MeasureBandwidth(path) delay ← MeasureDelay(path) utilization ← MeasureUtilization(path) Store bandwidth, delay, utilization IF LinkFailed(path) OR utilization > 80 THEN score[path] ← -∞ ELSE normalizedBandwidth ← Normalize(bandwidth) normalizedDelay ← NormalizeInverse(delay) normalizedUtilization ← NormalizeInverse(utilization) score[path] ← (0.50 × normalizedBandwidth) + (0.30 × normalizedDelay) + (0.20 × normalizedUtilization) END IF END FOR bestPath ← PathWithHighestScore(score) IF bestPath does not exist THEN DISPLAY "No healthy path available." ALERT administrator ELSE IF CurrentPath is failed OR CurrentPath is overloaded THEN SwitchTrafficTo(bestPath) ALERT administrator ELSE Continue using bestPath END IF END IF WAIT monitoringInterval END WHILE END

Explanation: The score considers multiple performance factors rather than shortest distance alone. Higher bandwidth improves the score, while lower delay and lower utilization are preferred. Failed or overloaded paths are excluded. Periodic recalculation allows the routing decision to adapt to changing traffic conditions, improving reliability, routing efficiency and fault tolerance.

Algorithm Design Summary

The three algorithms use validation, arrays/lists, loops, conditional decisions, monitoring intervals, alerts and calculated metrics. These control structures make the solutions suitable for systematic network-performance monitoring and simulation.